DB25 DCE Cable Examples

The following are examples of cable configurations for connecting terminals, printers and modems to DB25 DCE (Female) connectors.

DB25 DCE Port pinout

Pin	Circuit	Function

1	Chassis

2	RXD		Receive Data

3	TXD		Transmit Data

4	RTS		Request To Send

5	CTS		Clear To Send

6	DSR		Data Set Ready

7	GND		Ground

8	DCD		Data Carrier Detect

20	DTR		Data Terminal Ready

22	RI		Ring Indicator

Loopback connector

For use with Specialix diagnostic utilities.

DB25 to DB25 Terminal cable configuration

For standard terminal operating at slow speeds or using software flow control.  A simple 3-wire connection can be used.

DB25						DB25 Terminal

RXD		2  ---------------------	2	TXD

TXD		3  ---------------------	3	RXD

GND		7  ---------------------	7	GND

DB25 to DB25 terminal with hardware flow control

For terminals operating at speeds high than 19200 baud or for terminals which do not support software flow control.

DB25						DB25 Terminal

RXD		2  ---------------------	2	TXD

TXD		3  ---------------------	3	RXD

RTS		4  ---------------------	20	DTR

CTS		5  ---------------------	5	CTS

GND		7  ---------------------	7	GND

DB25 to DB25 terminal using the modem device

Using the modem device on a local connection ensures that the login process is killed when the terminal is switched off.  This is achieved by wiring the terminals RTS or DTR to the DB25 DCD.

DB25						DB25 Terminal

RXD		2  ---------------------	2	TXD

TXD		3  --------------------	3	RXD

GND     	7  ---------------------	7	GND

DCD		8  ---------------------	20	DTR or   4 RTS

DB25 to DB25 terminal using the modem device and hardware flow control

Using the modem device on a local connection ensures that the login process is killed when the terminal is switched off.  This is achieved by wiring the terminals RTS to the DB25 DCD.

DB25						DB25 Terminal

RXD		2  --------------------	2	TXD

TXD		3  --------------------	3	RXD

RTS		4  --------------------	20	DTR

GND		7  --------------------	7	GND

DCD		8  --------------------	4	RTS

This example assumes that DTR on the terminal is being used for hardware flow control.  If RTS is used for hardware flow control connect DTR on the terminal to DCD on the DB25 socket and RTS on the terminal to RTS on the DB25 socket.

DB25 to DB9 PC Com Port configuration

For standard terminal emulation operating at slow speeds or using software flow control.  A simple 3-wire connection can be used.

DB25						DB9 Com Port

RXD		2  --------------------	3	TXD

TXD		3  --------------------	2	RXD

GND		7  --------------------	5	GND

DB25 to DB25 modem cable configuration

DB25						DB25 Modem

RXD		2  ---------------------	3	RXD

TXD		3  ---------------------	2	TXD

RTS		4  ---------------------	5	CTS

CTS		5  ---------------------	4	RTS

DSR		6  ---------------------	20	DTR

GND		7  ---------------------	7	GND

DCD		8  ---------------------	8	DCD

DTR		20 --------------------	6	DSR

# DB29 to DB9 Modem Cable

Pinout reported ok.

DB9 Modem		DB25 TA8

3 ----------------------- 	3 TXD

2 -----------------------	2 RXD

8 -----------------------	4 RTS

7 -----------------------	5 CTS

4 -----------------------	6 DSR

5 -----------------------	7 GND

1 -----------------------	8 DCD

6 -----------------------	20 DTR

9			22 RI   Not needed.

DB25 to DB25 ( DCE ) null modem cable configuration

DB25						DB25 Modem

RXD		2 ---------------------	3	RXD

TXD		3 ---------------------	2	TXD

RTS		4 ---------------------	5	CTS

CTS		5 ---------------------	4	RTS

DSR		6 ---------------------	20	DTR

DCD		8 -/

---  8	DCD

DTR		20 -----------------/	6	DSR

GND		7  --------------------- 	7	GND

DB25 to DB25 Serial Printer cable using software flow control

DB25						DB25 Serial Printer

RXD		2  ---------------------	2	TXD

TXD		3  ---------------------	3	RXD

GND		7  ---------------------	7	GND

DB25 to DB25 Serial Printer cable using hardware flow control

This example is for a printer using the DTR pin for hardware flow control.

DB25						DB25 Serial Printer

RXD		2  ----------------------	2	TXD

TXD		3  ----------------------	3	RXD

GND		7  ----------------------	7	GND

RTS		4  ----------------------	20	DTR