The I/O8+ ports are wired for RS232 with RJ12 connectors. The pinouts of these are:

# I/08+ RJ12 Pinouts

Pin		Signal		Description

1		DCD		Data Carrier Detect (IN)

2		RXD		Receive Data (IN)

3		DTR/RTS	Data Terminal Ready/ Request To Send (OUT)

4		GND		Ground

5		TXD		Transmit Data (OUT)

6		CTS		Receive Hardware Flow Control

- If you only need TXD and RXD (i.e. you don't need modem control) you can connect RJ11 connectors to the I/O8+.

- Pin 1 (DCD) - modem connections only. The modem raises this signal to tell the I/O8+ that a carrier has been detected.

- Pin 3 (DTR/RTS) - if the port is used for a modem connection, this signal acts as DTR, i.e. it will be dropped when the I/O8+ wants the modem to disconnect. If the port is being used for a local connection with hardware flow control, the signal acts as RTS, i.e. it is normally high, but if the input buffers become full it will be dropped to tell the other end to stop sending data.

- Pin 6 (CTS) - this signal is only used when hardware handshaking is set. It is an input to the I/O8+ and is normally kept high. The device at the other end of the line will drop the signal if it wants the I/O8+ to  stop sending data.

# Loopback connector

RJ12

DCD	1 _

DTR	3_|

CTS	6_|

RXD	2_

TXD	5_|

# RJ12 to RJ12 converters

Simple  terminal connection:

RJ12			RJ12

RXD	2  <--------	TXD

I/O8+		GND	4  ----------	GND	Terminal

TXD	5  -------->	RXD

Terminal using hardware flow control:

RJ12			RJ12

RXD	2  <--------	TXD

DTR/RTS	3  -------->	CTS

I/O8+		GND	4  ----------	GND	Terminal

TXD	5  -------->	RXD

CTS	6  <--------	RTS

- Here pin 3 on the I/O8+ acts as an input hardware flow control pin. The pins used for hardware flow control may vary from terminal to terminal but pin 6 (output hardware flow control) on the I/O8+ port must be connected to the pin on the terminal which indicates that its buffer is full.

- Bi-directional hardware flow control is not supported under ISC UNIX. However, by using the modem device (ttyI**) you can obtain output hardware flow control.

- Some terminals need additional signals on the connector. These may not be supported by the I/O8+ or your cable. The normal way to overcome this is to loop back one of the output lines from the terminal into the required input. Refer to the documentation supplied with your terminal, or the supplier of the terminal, for information on which loopbacks, if any, are required.

Modem connection:

RJ12			RJ12

DCD	1  <--------	DCD

RXD	2  <--------	TXD

DTR/RTS	3  -------->	DTR

I/O8+		GND	4  ----------	GND	Modem

TXD	5  -------->	RXD

CTS	6  <--------	RTS

On modem devices, pin 3 on the I/O8+ is configured as the modem control pin (DTR) and there is no hardware flow control on input (RTS).

Serial printer connection:

Connections for a serial printer depend on whether the printer supports independent hardware flow control or not. Under ISC UNIX, which doesn't support independent hardware flow control, make sure the modem device is used for the port. Connections for a printer with hardware flow control are:

RJ12			RJ12

RXD	2  <--------	TXD

I/O8+		GND	4  ----------	GND	Printer

TXD	5  -------->	RXD

CTS	6  <--------	DTR

# RJ12 to DB25 Converters

You can purchase Specialix RJ12 to DB25 converter cables from your supplier. If you want to make your own, connect the  following pins:

Simple  terminal connection:

RJ12			DB25

RXD	2  <--------  2	TXD

I/O8+		GND	4  ----------  7	GND	Terminal

TXD	5  -------->  3	RXD

RJ12 to DB25 (DCE) converter CIO8+F:

DCD	1  <--------  8	DCD

RXD	2  <--------  2	TXD

DTR/RTS	3  -------->  5	CTS

|_>  6	DSR

RJ12		GND  	4  ----------  7	GND	DB25

TXD	5  --------->  3	RXD

CTS	6  <---------  4	RTS

RJ12 to DB25 (DTE) converter CIO8+M:

DCD	1  <--------  8	DCD

RXD	2  <--------  3	RXD

RTS/DTR	3  -------->  4 	RTS

|_>  20	DTR

RJ12		GND  	4  ----------  7	GND	DB25

TXD	5  --------->  2	TXD

CTS	6  <--------  5	CTS

# RJ12 to DB9 Converters

RJ12 to DB9 (DCE Male) converter:

DCD	1  <--------  1	DCD

RXD	2  <--------  2	RXD

DTR/RTS	3  -------->  4	DTR

|_>  7	RTS

RJ12		GND	4  ----------  5	GND	DB9

TXD	5  -------->  3	TXD

CTS	6  <--------  8	CTS

RJ12 to DB9 (DTE Female) PC converter:

DCD	1  <--------  1	DCD

RXD	2  <--------  3	TXD

DTR/RTS	3  -------->  6	DSR

|_>  8	CTS

RJ12		GND	4  ----------  5	GND	DB9

TXD	5  -------->  2	RXD

CTS	6  <--------  7	RTS