Perle IOLAN+ RJ45 RS232 serial cable diagrams

1	(DCD)	In

2	(RTS) 	Out

3	(DSR)	In

4	(TXD)	Out

5	(RXD) 	In

6	(GND)

7	(CTS)	In

8	(DTR)	Out

Terminal diagram Software Flow control

IOLAN+ RJ45			Terminal DB25

4  (TXD) -----------------------------	3  (RXD)

5  (RXD) -----------------------------	2  (TXD)

6  (GND) -----------------------------	7  (Gnd)

Printer Cable diagram DTR flow control

IOLAN+ RJ45		Printer DB25

4 (txd)	----------------	3 (rxd)

5 (rxd)	----------------	2 (txd)

6 (gnd)	----------------	7 (gnd)

7 (cts)	----------------	20 (dtr)

Note: Perle manufactures this adapter type. Part# 04006933 RJ45 to Printer DB25 Male.

Terminal or PC cable diagrams Hardware Flow control

IOLAN+ RJ45			Terminal or PC DB25

3  (DSR) -----------------------------	20  (DTR)

2  (RTS) -----------------------------	5  (CTS)

4  (TXD) -----------------------------	3  (RXD)

5  (RXD) -----------------------------	2  (TXD)

6  (GND) -----------------------------	7  (Gnd)

7 (CTS)  -----------------------------	4  (RTS)

8 (DTR) -----------------------------	6  (DSR)

Note: Perle manufactures this adapter. Part# 04006930 RJ45 to PC DB25 Female.

IOLAN+ RJ45			Terminal or PC DB9

3  (DSR)  -----------------------------	4  (DTR)

2  (RTS) ------------------------------	8  (CTS)

4  (TXD) ------------------------------	2  (RXD)

5  (RXD) ------------------------------	3 (TXD)

6  (Gnd) -------------------------------	5 (Gnd)

7  (CTS) ------------------------------	7  (RTS)

8 (DTR) -------------------------------	6  (DSR)

Note: Perle manufactures this adapter. Part# 04006931 RJ45 to PC DB9 Female.

Modem cable diagram

IOLAN+ RJ45		Modem DB25

1 (DCD) ------------------	8  (DCD)

2 (RTS)  ------------------	4  (RTS)

3 (DSR)  ------------------	6  (DSR)

4 (TXD)  ------------------	2  (TXD)

5 (RXD)  ------------------	3  (RXD)

6 (Gnd)   ------------------	7  (Gnd)

7 (CTS)  ------------------	5  (CTS)

8 (DTR)  ------------------	20 (DTR)

Note: Perle manufactures this adapter and cable type. Part# 04006932 RJ45 to DB25 Male (adapter) or cable w/adapter Part# 04001480 RJ45 to DB25 Male (Pak of 4)

Sun Sparc console diagram

IOLAN+ RJ45			Sun DB25

2  (RTS) 	----------------------------->	5  (CTS)

3  (DSR)	<-----------------------------	20  (DTR)

4  (TXD) 	----------------------------->	3  (RXD)

5  (RXD) 	<-----------------------------	2  (TXD)

6  (GND) 	----------------------------->	7  (Gnd)

7 (CTS)  	<-----------------------------	4  (RTS)

8 (DTR) 	----------------------------->	6  (DSR)

|--------->	8  (DCD)

Sun Sparc Netra and Cisco RJ45 console

IOLAN+ RJ45			RJ45 console

2  (RTS) 	----------------------------->	8  (CTS)

3  (DSR)	<-----------------------------	2  (DTR)

4  (TXD) 	----------------------------->	6  (RXD)

5  (RXD) 	<-----------------------------	3  (TXD)

6  (GND) 	----------------------------->	4  (Gnd)

7 (CTS)  	<-----------------------------	1  (RTS)

8 (DTR) 	----------------------------->	7  (DSR)

IBM RS6000

IOLAN+ RJ45			RS6000 DB9

2  (RTS) 	------------------------------>	8  (CTS)

3  (DSR)  	<------------------------------	1  (DCD)

4  (TXD) 	------------------------------>	2  (RXD)

5  (RXD) 	<------------------------------	3 (TXD)

6  (Gnd) 	--------------------------------	5 (Gnd)

7  (CTS) 	<------------------------------	7  (RTS)