# Pin	Circuit		Function

1	DCD	Input	Data Carrier Detect

2	DSR	Output	Data Set Ready

3	DTR	Input	Data Terminal Ready

4	S/GND		Signal Ground

5	TXD	Output	Transmit Data

6	RXD	Input	Receive Data

7	CTS	Output	Clear To Send

8	RTS	Input	Request To Send

Shield	P/GND		Protective (Chassis) Ground

# Loopback cable

DCD	1  __

DSR	2  __|

DTR	3  __|

TXD	5  __

RXD	6  __|

CTS	7  __

RTS	8  __|

# RJ45 to DB25 JS8500/LS2000/CS9000 console cable configuration

RJ45			DB25 console

DTR	2  --------------------- 20	DTR

GND	4  --------------------- 7	GND

TXD	5  --------------------- 2	TXD

RXD	6  --------------------- 3	RXD

Do not connect other signals.

# RJ45 to DB9 JS4000 console cable configuration

RJ45			DB9 Console port

S/GND	4  ---------------------	5	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

Do not connect other signals

# RJ45 to DB9 PC Com Port configuration

RJ45			DB9 Com Port

DCD	1  ---------------------	1	DCD

DSR	2  ---------------------	6	DSR

DTR	3  ---------------------	4	DTR

S/GND	4  ---------------------	5	S/GND

TXD	5  ---------------------	2	RXD

RXD	6  ---------------------	3	TXD

CTS	7  ---------------------	8	CTS

RTS	8  ---------------------	7	RTS

# RJ45 to RJ45 Null Modem Cable

RJ45				RJ45

DCD	1 ------------------------  	3	DTR

DSR	2 __|

DTR	3 ------------------------ 	1	DCD

|___	2	DSR

S/GND	4 ------------------------	4	S/GND

TXD	5 ------------------------	6	RXD

RXD	6 ------------------------	5	TXD

CTS	7 ------------------------	8	RTS

RTS	8 ------------------------	7	CTS

# RJ45 to DB9 null modem cable

RJ45			Server DB9

TXD	5  ---------------------	2	RXD

RXD	6  ---------------------	3	TXD

CTS	7  ---------------------	8	CTS

RTS	8  ---------------------	7	RTS

GND	4  ---------------------	5	GND

DCD	1  ---------------------	6	DSR

DSR	2  __|

DTR	3  ---------------------	1	DCD

|____	4	DTR

# RJ45 to DB25 Terminal cable configuration

For standard terminal operating at slow speeds or using software flow control.  A simple 3-pin connection can be used.

RJ45			DB25 Terminal

S/GND	4  ---------------------	7	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

# RJ45 to DB25 terminal with hardware flow control

For terminals operating at speeds high than 9600 baud or for terminals which do not support software flow control. Terminal supports DTR flow control.

RJ45			DB25 Terminal

S/GND	4  ---------------------	7	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

CTS	7  ---------------------	5	CTS

RTS	8  ---------------------	20	DTR

# RJ45 to DB25 terminal using the modem device

Using the modem device on a local connection ensures that the login process is killed when the terminal is switched off.

This is achieved by wiring the terminals RTS or DTR to the RJ45 DCD.

RJ45			DB25 Terminal

DCD	1  ---------------------	20	DTR or   4 RTS

S/GND	4  ---------------------	7	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

# RJ45 to DB25 terminal using the modem device and hardware flow control

Using the modem device on a local connection ensures that the login process is killed when the terminal is switched off.  This is achieved by wiring the terminals RTS to the RJ45 DCD.

RJ45			DB25 Terminal

DCD	1  ---------------------	20	DTR or   4 RTS

S/GND	4  ----------------------	7	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

This example assumes that DTR on the terminal is being used for hardware flow control.  If RTS is used for hardware flow control connect DTR on the terminal to DCD on the RJ45 and RTS on the terminal to RTS on theRJ45

# RJ45 to DB25 modem cable configuration

RJ45			DB25 Modem

DCD	1 ---------------------	8	DCD

DSR	2 ---------------------	20	DTR

DTR	3 ---------------------	6	DSR

S/GND	4 ---------------------	7	S/GND

TXD	5 ---------------------	2	TXD

RXD	6 ---------------------	3	RXD

CTS	7 ---------------------	4	RTS

RTS	8 ---------------------	5	CTS

# DB25 to DB9 modem cable configuration

RJ45			DB9 Modem

DCD	1 ---------------------	1	DCD

DSR	2 ---------------------	4	DTR

DTR	3 ---------------------	6	DSR

S/GND	4 ---------------------	5	S/GND

TXD	5 ---------------------	3	TXD

RXD	6 ---------------------	2	RXD

CTS	7 ---------------------	7	RTS

RTS	8 ---------------------	8	CTS

# RJ45 to DB25 Serial Printer cable using software flow control

RJ45			DB25 Serial Printer

S/GND	4  ---------------------	7	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

# RJ45 to DB25 Serial Printer cable using hardware flow control

This example is for a printer using the DTR pin for hardware flow control.

RJ45			DB25 Serial Printer

S/GND	4  ---------------------	7	S/GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

RTS	8  --------------------	20	DTR

# RJ45 to DB9 Serial Printer cable using hardware flow control

This example is for a printer using the DTR pin for hardware flow control.

RJ45			DB9 Serial Printer

S/GND	4  ---------------------	5	S/GND

TXD	5  ---------------------	3	TXD

RXD	6  ---------------------	2	RXD

RTS	8  ---------------------	4	DTR

# RJ45 to DB9 IBM RS6000 Com Port configuration

RJ45			DB9 Com Port

DSR	2  ---------------------	1	DCD

RXD	6  ---------------------	3	TXD

TXD	5  ---------------------	2	RXD

S/GND	4  ---------------------	5	S/GND

RTS	8  ---------------------	7	RTS

CTS	7  ---------------------	8	CTS

# RJ45 to DB25 Sun Server

Here are the 2 main cable pinouts required to connect to Sun servers and workstations.

RJ45			DB25 serial

DTR	3  ---------------------	20	DTR

DSR	2  ---------------------	6	DSR

GND	4  ---------------------	7	GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

CTS	7  ---------------------	5	CTS

|________	8	DCD

RTS	8  ---------------------	4	RTS

# RJ45 to Sun server Netra Port RJ45

RJ45			RJ45 serial

DSR	2  ---------------------	7	DSR

DTR	3  ---------------------	2	DTR

GND	4  ---------------------	4	GND

TXD	5  ---------------------	6	RXD

RXD	6  ---------------------	3	TXD

CTS	7  ---------------------	8	CTS

RTS	8  ---------------------	1	RTS

# RJ45 to Sun server Port DB9

RJ45			DB9 serial

DTR	3	------------	4	DTR

DSR	2	------------	6	DSR

|___	1	DCD

GND	4	------------	5	GND

TXD	5	------------	2	RXD

RXD	6	------------	3	TXD

CTS	7	------------	8	CTS

RTS	8	------------	7	RTS

RJ45 to Nokia 400/440 DB9 Com Port

Use RJ45 to DB9 PC pinout (or adapter)

# RJ45 to RJ45 Cisco cable with hardware flow control

This example supports both DTR/DSR and RTS/CTS signalling for both routers and switches.

RJ45			RJ45 Console port

DSR	2 ---------------------	7	DSR

DTR	3 ---------------------	2	DTR

S/GND	4 ---------------------	4	GND

TXD	5 ---------------------	6	RXD

RXD	6 ---------------------	3	TXD

CTS	7 ---------------------	8	CTS

RTS	8 ---------------------	1	RTS

# RJ45 to DB25 Perle P series router console

RJ45			DB25 console

DTR	2  ---------------------	20	DTR

GND	4  ---------------------	7	GND

TXD	5  ---------------------	3	RXD

RXD	6  ---------------------	2	TXD

# RJ45 to RJ45 Perle P series router console

RJ45			RJ45 Console port

DSR	2 ---------------------	3	DTR

S/GND	4 ---------------------	4	GND

TXD	5 ---------------------	6	RXD

RXD	6 ---------------------	3	TXD

# RJ45 to DB25 Nortel switch

RJ45			DB25

DCD	1 ---------------------	4	RTS

S/GND	4 ---------------------	7	S/GND

TXD	5 ---------------------	3	RXD

RXD	6 ---------------------	2	TXD

RTS	8 ---------------------	20	DTR

RJ45  to  Foundry Big Iron Switch DB9 serial port

RJ45				DB9

S/GND	4  ---------------------	5	S/GND

TXD     	5  ---------------------	3	RXD

RXD     	6  ---------------------	2	TXD

CTS     	7  ---------------------	8	RTS

RTS     	8  ---------------------	7	CTS