RJ45 Non-Standard Cabling Examples

The following are examples of cable configurations for connecting terminals, printers and modems to RJ45   (Non-Standard or  Unshielded ) connectors.

RJ45 Port pinout

Pin	Circuit	Function

1	DSR		Data Set Ready

2	DCD		Data Carrier Detect

3	RXD		Receive Data

4	CTS		Clear To Send

5	GND		Ground

6	TXD		Transmit Data

7	DTR		Data Terminal Ready

8	RTS		Request To Send

Loopback connector

For use with Specialix diagnostic utilities.

RJ45 to DB25 Terminal cable configuration

For standard terminal operating at slow speeds or using software flow control.  A simple 3-wire connection can be used.

RJ45						DB25 Terminal

RXD		3  ---------------------	2	TXD

TXD		6  ---------------------	3	RXD

S/GND	5  ---------------------	7	S/GND

RJ45 to DB25 terminal with hardware flow control

For terminals operating at speeds high than 19200 baud or for terminals which do not support software flow control.

RJ45						DB25 Terminal

RXD		3  ---------------------	2	TXD

TXD		6  ---------------------	3	RXD

RTS		8  ---------------------	20	DTR

CTS		4  ---------------------	5	CTS

S/GND	5  ---------------------	7	S/GND

RJ45 to DB25 terminal using the modem device

Using the modem device on a local connection ensures that the login process is killed when the terminal is switched off.  This is achieved by wiring the terminals RTS or DTR to the RJ45 DCD.

RJ45						DB25 Terminal

RXD		3  ---------------------	2	TXD

TXD		6  ---------------------	3	RXD

S/GND     	5  ----------------------	7	S/GND

DCD		2  ----------------------	20	DTR or   4 RTS

RJ45 to DB25 terminal using the modem device and hardware flow control

Using the modem device on a local connection ensures that the login process is killed when the terminal is switched off.  This is achieved by wiring the terminals RTS to the RJ45 DCD.

RJ45						DB25 Terminal

RXD		3  ---------------------	2	TXD

TXD		6  ---------------------	3	RXD

RTS		8  ---------------------	20	DTR

S/GND	5  ---------------------	7	S/GND

DCD		2  ---------------------	4	RTS

This example assumes that DTR on the terminal is being used for hardware flow control.  If RTS is used for hardware flow control connect DTR on the terminal to DCD on the RJ45 socket and RTS on the terminal to RTS on the RJ45 socket.

RJ45 to DB9 PC Com Port configuration

For standard terminal emulation operating at slow speeds or using software flow control.  A simple 3-wire connection can be used.

RJ45						DB9 Com Port

RXD		3  ---------------------	3	TXD

TXD		6  ---------------------	2	RXD

S/GND	5  ---------------------	5	S/GND

RJ45 to DB25 modem cable configuration

RJ45						DB25 Modem

RXD		3 ----------------------	3	RXD

TXD		6 ----------------------	2	TXD

RTS		8 ----------------------	5	CTS

CTS		4 ----------------------	4	RTS

DSR		1 ----------------------	20	DTR

S/GND	5 ----------------------	7	S/GND

DCD		2 ----------------------	8	DCD

DTR		7 ----------------------	6	DSR

RJ45 to DB25 Serial Printer cable using software flow control

RJ45						DB25 Serial Printer

RXD		3  ---------------------	2	TXD

TXD		6  ---------------------	3	RXD

S/GND	5  ---------------------	7	S/GND

RJ45 to DB25 Serial Printer cable using hardware flow control

This example is for a printer using the DTR pin for hardware flow control.

RJ45						DB25 Serial Printer

RXD		3  ---------------------	2	TXD

TXD		6  ---------------------	3	RXD

S/GND	5  ---------------------	7	S/GND

RTS		8  ---------------------	20	DTR