# Perle Router Cables

This document describes the cables used by the Perle family of routers. These cables are fully described in the respective product manuals.

## Perle P840

| Required Cables | Description |
| --- | --- |
| Console cable | RJ45 <—> DB9 (F) (See Cable 1) |
| ISDN | RJ45 <—> RJ45 |

## Perle P850

| Required Cables | Description |
| --- | --- |
| Console cable | RJ45 <—> DB9 (F) (See Cable 1) |
| V.35 | DB25 (M) <—> V.35 (M) (See Cable 2) |
| V.24 | DB25 (M) <—> DB25 (M) (straight through) |
| X.21 | DB25 (M) <—> DB15 (M) (See Cable 3) |

## Perle 1700

| Required Cables | Description |
| --- | --- |
| Console cable | DB25 (M) <—> DB9 (F) (standard mapping) |
| V.24 | DB25 (M) <—> DB25 (M) (straight through) |
| V.35 | DB25 (M) <—> V.35 (M) (See Cable 4) See Part # 04010230 |
| X.21 | DB15 (M) <—> DB15 (M) (straight through) |
| RS422 | DB25 (M) <—> DB25 (M) (straight through) |
| ISDN S, U | RJ45 <—> RJ45 |
| CSU/DSU | RJ45 <—> RJ45 |

### Notes:

- The V.35 connector is designated as M.34

- Cable diagrams and additional details are in the appendices of the User and System Administration Guides.

## Perle P2600

| Required Cables | Description |
| --- | --- |
| Console cable | DB25 (M) <—> DB9 (F) (Standard mapping) |
| V.24 | DB25 (M) <—> DB25 (M) (straight through) |
| V.35 | DB25 (M) <—> V.35 (M) (See Cable 5) See Part # 04010230 |
| X.21 | DB15 (M) <—> DB15 (M) (straight through) |
| RS422 | DB25 (M) <—> DB25 (M) (straight through) |
| ISDN S, U | RJ45 <—> RJ45 |
| CSU/DSU | RJ45 <—> RJ45 |

## Cable 1: Console: RJ45 <—> DB9 (F)

| RJ45 |  | DB9 (F) |
| --- | --- | --- |
| 2 | --------------------- | 8 |
| 3 | --------------------- | 4 |
| 4 | --------------------- | 5 |
| 5 | --------------------- | 2 |
| 6 | --------------------- | 3 |
| 7 | --------------------- | 6 |
| 8 | --------------------- | 1 |

## Cable 2: V.35: DB25 (M) <—> M.34 (M)

Note: Perle850 ONLY

| DB25 |  | M.34 (M) |
| --- | --- | --- |
| 1 | --------------------- | A |
| 2 | --------------------- | P |
| 3 | --------------------- | R |
| 4 | --------------------- | C |
| 5 | --------------------- | D |
| 6 | --------------------- | E |
| 7 | --------------------- | B |
| 8 | --------------------- | F |
| 9 | --------------------- | X |
| 11 | --------------------- | W |
| 12 | --------------------- | AA |
| 14 | --------------------- | S |
| 15 | --------------------- | Y |
| 16 | --------------------- | T |
| 17 | --------------------- | V |
| 18 | --------------------- | L |
| 20 | --------------------- | H |
| 21 | --------------------- | N |
| 24 | --------------------- | U |
| 25 | --------------------- | NN |

Notes:

- The connecting cable must be shielded.

- The following circuits should be connected to twisted pairs within the connecting cable:

2 – 14; 3 – 16; 9 – 17; 11 – 24; 12 – 15;

## Cable 3: X.21: DB25 (M) <—> DB15 (M)

Note: Perle P1700 ONLY

| DB25 |  | DB15 (M) |
| --- | --- | --- |
| 1 | --------------------- | 1 |
| 2 | --------------------- | 2 |
| 3 | --------------------- | 4 |
| 7 | --------------------- | 8 |
| 8 | --------------------- | 5 |
| 10 | --------------------- | 12 |
| 12 | --------------------- | 13 |
| 14 | --------------------- | 9 |
| 15 | --------------------- | 6 |
| 16 | --------------------- | 11 |
| 20 | --------------------- | 3 |
| 23 | --------------------- | 10 |

## Cable 4: V.35: DB25 (M) <—> M.34 (M)

| DB25 (M) |  | M.34 (M) |
| --- | --- | --- |
| 1 | --------------------- | A |
| 4 | --------------------- | C |
| 6 | --------------------- | E |
| 7 | --------------------- | B |
| 8 | --------------------- | F |
| 9 | --------------------- | P |
| 10 | --------------------- | S |
| 11 | --------------------- | R |
| 12 | --------------------- | T |
| 14 | --------------------- | V |
| 16 | --------------------- | X |
| 18 | --------------------- | U |
| 19 | --------------------- | W |
| 20 | --------------------- | H |
| 22 | --------------------- | J |
| 23 | --------------------- | Y |
| 25 | --------------------- | a |

Notes:

- The connecting cable must be shielded.

- The following circuits should be connected to twisted pairs within the connecting cable:

9 – 10; 11 – 12; 14 – 16; 18 – 19; 23 - 25